Adding the MCS7840 drivers to the Platform

Introduction:

This document provides the information about how to add/include MCS7840 device drivers to a windows CE .NET 5.0 platform.

The following steps explains how to add/include the  MCS7840 device drivers to Windows CE .NET 5.0 platform.

- Create and Build a CEPC platform

- After building the platform add the following entries in the platform.reg file

[HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30784\255_0_255\Default\Mos40UsbSerialClass]

"Dll"="SpecCe50.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\Mos40UsbSerialClass]

"Dll"="SpecCe50.DLL"

- Add the following entries in the platform.bib file

SpecCe50.dll  $(_FLATRELEASEDIR)\SpecCe50.dll               NK

- Copy the SpecCe50.dll in to the following folder.

Drive:>\WINCE500\PUBLIC\CEPlatform\RelDir\CEPC_X86Release

- Then build the platform to effect the above changes.

This platform image contains the  MCS7840 device driver and you can connect  MCS7840 to the target machine and configure after downloading the platform OS image.